Section 08 14 16

# Flush Wood Door – (Wood Veneer)

Part 1	General

  - Section includes non-rated and fire rated flush wood doors.

- Flush

- Glazed

- Limited applied moldings

  - Related Sections

  - Section 06 20 00	Finish Carpentry

  - Section 06 40 00	Interior Architectural Woodwork

  - Section 08 11 00	Metal doors and frames

  - Section 08 70 00	Finish Hardware

  - Section 08 81 00	Glazing

  - References

- NFPA 80 Standard for Fire Doors and Other Opening Protectives (Current Edition)

- NFPA 252 Standard Methods of Fire Tests of Door Assemblies

- UBC STD 7-2 (1997) Fire Tests of Door Assemblies

- UL10C Positive Pressure Fire Tests of Door Assemblies

- NFPA 105 Standard for the Installation of Smoke Door Assemblies and Other Opening Protectives (Current Edition)

- ANSI/WDMA I.S. 1A -11 Industry Standard for Architectural Wood Flush Doors

- ASTM E779-10 Determining Air Leakage Rate by Fan Pressurization

- ASTM E-90-09 Laboratory Measurements of Airborne Sound Transmission Loss of Building Partition Elements

- ASTM E413-10 Classification for Rating Sound Insulation

  - Quality Assurance

- All doors must be supplied through one manufacturer with a minimum of five years documented experience in the fabrication of WDMA IS 1A compliant doors.

- Meet or exceed ANSI/WDMA I.S.-1A-04 Extra Heavy-Duty Construction

- Fire doors: tested and listed in accordance with UL10C, NFPA 252, UBC STD 7-2 	(1997)

  - Delivery, storage, handling and site conditions:

    - Deliver, store, protect and handle doors under provisions of ANSI/WDMA I.S. 1A-2011 J-1 and manufacturer’s instructions.

    - Do not deliver or install doors until spaces are enclosed and watertight, wet work in spaces is complete and dry, and HVAC system is operating and maintaining ambient temperature & humidity conditions at occupancy levels during the remainder of the construction period (relative humidity is not be less than 25%, or exceeds 55%).

    - Inspect doors for damage prior to accepting job site delivery

    - Do not store doors in areas subject to direct sunlight

    - Certain wood species are light sensitive.  Protect doors from exposure to natural and artificial light after delivery

    - Store all doors in horizontal flat position prior to installation

  - Warranty:

- Provide manufacturer’s warranty to comply with the following:

  - Interior solid core doors; “Limited Life of Original Installation”, including prior 	factory approved and authorized compensation for

- Delivery and re-hanging if door(s) do not comply with warranty 	tolerance standards.

- Re-finishing if door(s) do not comply with warranty tolerance standards.

  - Manufacturer’s warranty excludes the appearance of field-finished door(s).

  - Submittals:

- Submit shop drawings: illustrate and identify door openings to include elevations, sizes, bevels, undercuts, types, fire ratings, swings and machining specifications.

- Submit two samples of stain and finish on the specified veneer for architectural approval prior to finishing.

- Submit manufacturer’s specification literature to assure product compliance.

- Submit copy of manufacturer’s limited lifetime warranty.

Part 2	Products / Materials

  - Door Manufacturers:

- Manufacturer: Lynden Door Inc. LD Series Construction or approved equal,

- Other Acceptable manufacturers:

  - Eggers Industries

  - Vancouver Door

  - Other:

- Substitutions are allowed only if approved by the architect prior to bid date

  - Door Construction:

- WDMA Custom Grade Extra Heavy-Duty Construction

- Non-rated and 20-minute rated: WDMA PC-5 or PC-7, LD-1 minimum grade (LD2000)

- Minimum 1-1/4” stiles prior to trim

- Minimum 2” top & bottom rails prior to trim

- Reinforced blocking required for all surface mounted hardware when thru bolts are not use in the installation of the hardware

- Stiles and rails securely bonded to the core with adhesive, then bonded assembly machine calibrated prior to application of face components to achieve minimal telegraphing per section T-1

- Construct 20 minute rated pairs so that no metal edge and/or astragal is required

- Non-rated and 20-minute rated with vision lite exceeding half of door height or 40% of core field or installed without hinges or pivots: WDMA SCLC-5 or SCLC-7 (LD3500)

  - Minimum 1-1/4” stiles prior to trim

  - Rails to be an integral component of the core

  - Stiles and rails securely bonded to the core with adhesive, then bonded assembly machine calibrated prior to application of face components to achieve minimal telegraphing per section T-1

  - Construct 20-minute rated pairs so that no metal edge and/or astragal is required

- Ventilated Door (VanAir VA7500) with an integrated air channel in a z-shaped profile and opposing ventilation openings routed into the front and back faces, and integrated acoustic baffling air channel

- 45, 60 & 90 minute fire rated: WDMA FD7 or FD5 Custom Grade (LD4500, LD6000, & LD9000)

  - Fabricate in accordance with manufacturer’s U.L or Warnock Hersey (ITS) listings.

  - Reinforced blocking required for all surface mounted hardware when thru bolts are not used in the installation of the hardware.

  - Construct pairs of doors so that no metal edge and/or astragal is required

- 20 Minute rated single swing doors to be supplied as Category ‘B’ construction.  See Section 08 70 00 to assure gasketing complies with fire door listing and ‘S’ label requirements

- 20 Minute rated pairs of door and all 45, 60, & 90 minute rated doors to be supplied as Category ‘A’ construction.  See Section 08 70 00 to assure gasketing complies with ‘S’ label requirements

  - Veneer:

- Specie: ___________________________ (Select Specie)

- Veneer Cut: Rotary / Plain Sliced / Quarter Sliced / Rift Cut (Select One)

- Veneer Match: Book Match / Slip Match (Select One)

- Veneer Assembly: Running Match

- Pair match at doors installed in pairs

- Set match at doors installed in sets of 4 or more leaves

  - Accessories:

- Wood Non-rated and 20-minute vision stop

1.	Wood lip stop (WVF-2 profile) compatible with face veneer

- 45, 60 & 90-minute approved and listed metal vision frames.

- Glazing of fire doors to be performed at factory or licensed facility.

- Factory machine doors for finish hardware (Ref. Sec. 08 70 00)

  - Include pilot holes for hinges

  - Include face holes for mortise and exit devices (excluding trim holes)

  - Include thru bolt holes for lever design cylindrical locks

  - Factory seal top and bottom rails

  - Finish: Factory Finish TR-8 UV Cured Acrylated Polyester Urethane Blend system 	containing no Volatile Organic Compounds (VOC’s)

- Stain Color (Select One)

  - Factory standard stain color, Sample #_____

  - Custom stain color, sample to be selected by the architect

- Architect must approve all finish samples prior to finishing.

- All factory finished doors to be individually poly-bagged for protection

Part 3	execution

  - Examination:

- Inspect doors for any damage prior to installation.  Do not install damaged doors as installation of damaged doors may void manufacturer’s warranty.

- Verify opening sizes and tolerances are acceptable and ready to receive this work.

- Do not install doors in frame openings that are not plumb or are out of tolerance for size or alignment.

  - Installation:

- Schedule installation of doors after finishing of walls and finish floor installation.

- Coordinate installation of doors with approved Finish Hardware and Frame Schedules

- Install doors in accordance with ANSI/WDMA IS-1A, NFPA 80, and NFPA 105

- Do not trim fire doors for width

- Provide 1/8” clearance at head, jambs and between pairs of doors

  - Adjusting

- Adjust operation of door for smooth and balanced movement.

- Do not remove hinge screws after initial insertion.  Shims used for alignment purposes must not be inserted between hinge and the door.

- Doors may be repaired or refinished if work complies with requirements and shows no evidence of repair or refinishing

- Replace doors that are damaged or do not comply with requirements.