Section 08 14 23.16

# Plastic-Laminate-Faced Wood Doors

Part 1	General

  - Section includes non-rated and fire rated flush wood doors.

- Flush

- Glazed

  - Related Sections

  - Section 06 20 00	Finish Carpentry

  - Section 06 40 00	Interior Architectural Woodwork

  - Section 08 11 00	Metal doors and frames

  - Section 08 70 00	Finish Hardware

  - Section 08 81 00	Glazing

  - References

- NFPA 80 Standard for Fire Doors and Other Opening Protectives (Current Edition)

- NFPA 252 Standard Methods of Fire Tests of Door Assemblies

- UBC STD 7-2 (1997) Fire Tests of Door Assemblies

- UL10C Positive Pressure Fire Tests of Door Assemblies

- NFPA 105 Standard for the Installation of Smoke Door Assemblies and Other Opening Protectives (Current Edition)

- ANSI/WDMA I.S. 1A -11 Industry Standard for Architectural Wood Flush Doors

- ASTM E779-10 Determining Air Leakage Rate by Fan Pressurization

- ASTM E-90-09 Laboratory Measurements of Airborne Sound Transmission Loss of Building Partition Elements

- ASTM E413-10 Classification for Rating Sound Insulation

  - Quality Assurance

- All doors must be supplied through one manufacturer with a minimum of five years documented experience in the fabrication of WDMA IS 1A compliant doors.

- Meet or exceed ANSI/WDMA I.S.-1A-04 Extra Heavy-Duty Construction

- Fire doors: tested and listed in accordance with UL10C, NFPA 252, UBC STD 7-2 (1997)

  - Warranty:

- Provide manufacturer’s warranty to comply with the following:

  - Interior solid core doors; “Limited Life of Original Installation”, including prior factory approved and authorized compensation for

- Delivery and re-hanging if door(s) do not comply with warranty 	tolerance standards.

- Re-finishing if door(s) do not comply with warranty tolerance standards.

  - Manufacturer’s warranty excludes the appearance of field-finished door(s).

  - Delivery, storage, handling and site conditions:

    - Deliver, store, protect and handle doors under provisions of ANSI/WDMA I.S. 1A-2011 J-1 and manufacturer’s instructions.

    - Do not deliver or install doors until spaces are enclosed and watertight, wet work in spaces is complete and dry, and HVAC system is operating and maintaining ambient temperature & humidity conditions at occupancy levels during the remainder of the construction period (relative humidity is not be less than 25%, or exceeds 55%).

    - Inspect doors for damage prior to accepting job site delivery

    - Do not store doors in areas subject to direct sunlight

    - Certain wood species are light sensitive.  Protect doors from exposure to natural and artificial light after delivery

    - Store all doors in horizontal flat position prior to installation

  - Submittals:

- Submit shop drawings: illustrate and identify door openings to include elevations, sizes, bevels, undercuts, types, fire ratings, swings and machining specifications.

- Submit two samples of stain and finish on the specified veneer for architectural approval prior to finishing.

- Submit manufacturer’s specification literature to assure product compliance.

- Submit copy of manufacturer’s limited lifetime warranty.

Part 2	Products / Materials

  - Door Manufacturers:

- Manufacturer: Lynden Door Inc. LD Series Construction or approved equal,

- Other Acceptable manufacturers:

  - Eggers Industries

  - Vancouver Door

  - Other:

- Substitutions are allowed only if approved by the architect prior to bid date

  - Door Construction:

- WDMA Custom Grade Extra Heavy-Duty Construction

- Non-rated and 20-minute rated: WDMA PC-3 or PC-5, LD-1 minimum grade (LD2000)

- Minimum 1-1/4” stiles prior to trim

- Minimum 2” top & bottom rails prior to trim

- Reinforced blocking required for all surface mounted hardware when thru bolts are not use in the installation of the hardware

- Stiles and rails securely bonded to the core with adhesive, then bonded assembly machine calibrated prior to application of face components to achieve minimal telegraphing per section T-1

- Construct 20 minute rated pairs so that no metal edge and/or astragal is required

- Non-rated and 20-minute rated with vision lite exceeding half of door height or 40% of core field or doors in high abuse areas (to be determined by the architect, see door schedule): WDMA SCLC-5 (LD3500)

  - Minimum 1-1/4” stiles prior to trim

  - Rails to be an integral component of the core

  - Stiles and rails securely bonded to the core with adhesive, then bonded assembly machine calibrated prior to application of face components to achieve minimal telegraphing per section T-1

  - Construct 20 minute rated pairs so that no metal edge and/or astragal is required

- Ventilated Door (VanAir VA7500) with an integrated air channel in a z-shaped profile and opposing ventilation openings routed into the front and back faces, and integrated acoustic baffling air channel

- 45, 60 & 90 minute fire rated: WDMA FD5 Custom Grade (LD4500, LD6000, & LD9000)

  - Fabricate in accordance with manufacturer’s U.L or Warnock Hersey (ITS) listings.

  - Reinforced blocking required for all surface mounted hardware when thru bolts are not used in the installation of the hardware.

  - Construct pairs so that no metal edge and/or astragal is required

- 20 Minute rated single swing doors to be supplied as Category ‘B’ construction.  See Section 08 70 00 to assure gasketing complies with fire door listing (Category G Gasketing) and ‘S’ label requirements (Category H Gasketing)

- 20 Minute rated pairs of door and all 45, 60, & 90 minute rated doors to be supplied as Category ‘A’ construction.  See Section 08 70 00 to assure gasketing complies with ‘S’ label requirements (Category H Gasketing)

  - HPDL:

- Manufacturer: ___________________________ (Select Manufacture of HPDL)

- Use General Purpose HPDL (.050 nominal thickness) with typical matte finish, color to be determined by architect

- Vertical edges to be of same material as faces.  Apply vertical HPDL edges after faces.

  - Machining:

    - Factory machine doors for frames and finish hardware

      - Locations of hardware prep will be coordinated between frame and door manufacturers

      - Include pilot holes for hinges

      - Include face holes for mortise and exit devices (excluding trim holes)

      - Include thru bolt holes for lever design cylindrical locks

    - Factory seal top and bottom rails

  - Light and Louver Openings – cutout and trim openings through doors to comply with NFPA 80 and manufacturer’s fire door listing and warranty:

- Rated and non-rated doors with vision lites are to be glazed with metal vision frames.  Metal vision frames to be supplied with manufacturer’s standard powder coat finish as approved by architect.

- Glazing of fire doors to be performed at factory or licensed facility.

- Non-fire doors may be glazed at factory.  Refer to Section 08 81 00 for glass requirements

Part 3	execution

  - Examination:

- Inspect doors for any damage prior to installation.  Do not install damaged doors as installation of damaged doors may void manufacturer’s warranty.

- Verify opening sizes and tolerances are acceptable and ready to receive this work.

- Do not install doors in frame openings that are not plumb or are out of tolerance for size or alignment.

  - Installation:

- Schedule installation of doors after finishing of walls and finish floor installation.

- Coordinate installation of doors with approved Finish Hardware and Frame Schedules

- Install doors in accordance with ANSI/WDMA IS-1A, NFPA 80, and NFPA 105

- Do not trim fire doors for width

- Provide 1/8” clearance at head, jambs and between pairs of doors

  - Adjusting

- Adjust operation of door for smooth and balanced movement.

- Do not remove hinge screws after initial insertion.  Shims used for alignment purposes must not be inserted between hinge and the door.

- Doors may be repaired or refinished if work complies with requirements and shows no evidence of repair or refinishing

- Replace doors that are damaged or do not comply with requirements.